Creating NIC Bonding:

- Add a new NIC if it does not exist

- Install bonding driver = modprobe bonding

- To list the bonding module info = modinfo bonding

You will see the driver version as seen below if the driver is installed and loaded

Create Bond Interface File

- vi /etc/sysconfig/network-scripts/ifcfg-bond0

- Add the following parameters

DEVICE=bond0

TYPE=Bond

NAME=bond0

BONDING_MASTER=yes

BOOTPROTO=none

ONBOOT=yes

IPADDR=192.168.1.80

NETMASK=255.255.255.0

GATEWAY=192.168.1.1

BONDING_OPTS=”mode=5 miimon=100”

- Save and exit the file

- The bonding options details are can be found on the following table

miimon

Specifies the MII link monitoring frequency in milliseconds. This determines how often the link state of each slave is inspected for link failures

Edit the First NIC File (enp0s3)

- vi /etc/sysconfig/network-scripts/ifcfg-enp0s3

- Delete the entire content

- Add the following parameters

TYPE=Ethernet

BOOTPROTO=none

DEVICE=enp0s3

ONBOOT=yes

HWADDR=”MAC from the ifconfig command”

MASTER=bond0

SLAVE=yes

- Save and exit the file

Create the Second NIC File (enp0s8) or Copy enp0s3

- vi /etc/sysconfig/network-scripts/ifcfg-enp0s8

- Add the following parameters

TYPE=Ethernet

BOOTPROTO=none

DEVICE=enp0s8

ONBOOT=yes

HWADDR=”MAC from the ifconfig command”

MASTER=bond0

SLAVE=yes

- Save and exit the file

Restart the Network Service

- systemctl restart network

Test and verify the configuration

- ifconfig 	or 	ifconfig | more

Use following command to view bond interface settings like bonding mode & slave interface

- cat /proc/net/bonding/bond0